# GenericCameraSensor

Same as generic sensor, but using espcam board.

Requires i2c pins and PIR pin for full data

#define PWDN_GPIO_NUM  32

#define RESET_GPIO_NUM -1

#define XCLK_GPIO_NUM  0

#define SIOD_GPIO_NUM  26

#define SIOC_GPIO_NUM  27

#define Y9_GPIO_NUM    35

#define Y8_GPIO_NUM    34

#define Y7_GPIO_NUM    39

#define Y6_GPIO_NUM    36

#define Y5_GPIO_NUM    21

#define Y4_GPIO_NUM    19

#define Y3_GPIO_NUM    18

#define Y2_GPIO_NUM    5

#define VSYNC_GPIO_NUM 25

#define HREF_GPIO_NUM  23

#define PCLK_GPIO_NUM  22

| GPIO | Input | Output | Notes | Pin Function |
| --- | --- | --- | --- | --- |
| 0 | pulled up | OK | outputs PWM signal at boot, must be LOW to enter flashing mode | CAM_XCLK_GPIO_NUM |
| 1 | TX pin | OK | debug output at boot | Serial0_TX |
| 2 | OK | OK | connected to on-board LED, must be left floating or LOW to enter flashing mode | SD_DATA0 |
| 3 | OK | RX pin | HIGH at boot | Serial0_RX |
| 4 | OK | OK |  | SD_DATA1/flashlight |
| 5 | OK | OK | outputs PWM signal at boot, strapping pin | CAM_Y2_GPIO_NUM |
| 6 | x | x | connected to the integrated SPI flash | NONE |
| 7 | x | x | connected to the integrated SPI flash | NONE |
| 8 | x | x | connected to the integrated SPI flash | NONE |
| 9 | x | x | connected to the integrated SPI flash | NONE |
| 10 | x | x | connected to the integrated SPI flash | NONE |
| 11 | x | x | connected to the integrated SPI flash | NONE |
| 12 | OK | OK | boot fails if pulled high, strapping pin | SD_DATA2 |
| 13 | OK | OK |  | SD_DATA3 |
| 14 | OK | OK | outputs PWM signal at boot | SD_CLK |
| 15 | OK | OK | outputs PWM signal at boot, strapping pin | SD_CMD |
| 16 | OK | OK |  | GPIO1 = PIR |
| 17 | OK | OK |  |  |
| 18 | OK | OK |  | CAM_Y3_GPIO_NUM |
| 19 | OK | OK |  | CAM_Y4_GPIO_NUM |
| 21 | OK | OK |  | CAM_Y5_GPIO_NUM |
| 22 | OK | OK |  | CAM_PCLK_GPIO_NUM |
| 23 | OK | OK |  | CAM_HREF_GPIO_NUM |
| 25 | OK | OK |  | CAM_VSYNC_GPIO_NUM |
| 26 | OK | OK |  | CAM_SIOD_GPIO_NUM |
| 27 | OK | OK |  | CAM_SIOC_GPIO_NUM |
| 32 | OK | OK |  | CAM_PWDN_GPIO_NUM |
| 33 | OK | OK |  | GPIO3 = PIR |
| 34 | OK |  | input only | CAM_Y8_GPIO_NUM |
| 35 | OK |  | input only | CAM_Y9_GPIO_NUM |
| 36 | OK |  | input only | CAM_Y6_GPIO_NUM |
| 39 | OK |  | input only | CAM_Y7_GPIO_NUM |

# 22June24

Using desksensor to see how the esp32 will react when wifi is not available, so the MAVLink device on the LED plane will work without wifi.

- Ideally, I likely want the wifi to backoff for 1 minute at first, then 10 minutes.